Intetics  ·  SmartLab / DataVest

DataVest Brew

Clickable prototype: build specification

Document date:  21 October 2026

Prepared by:  Dzmitry, Intetics

Classification:  Internal

Exported from the Claude Doc “Brewery Process Intelligence Platform”, tab “Prototype spec”. Diagrams are images; text and tables are editable.

Contents

Right-click and choose Update Field to build the table of contents.

# DataVest Brew — Prototype specification

This is the build brief for a clickable prototype of DataVest Brew. The prototype runs on synthetic data and is used in discovery calls with E+H and breweries. It shows all six modules; integrations are simulated.

## Purpose and scope

Goal: let a brewer, a lab manager and an E+H representative see, in 15 minutes, how the product connects equipment, the lab and production, and what it predicts.

In scope:

- the six modules with demo-ready screens;

- a simulated edge gateway with OPC UA, Modbus TCP and Profinet feeds;

- a simulated Netilion feed for the QWX43 and an E+H feed for pH, oxygen and CO₂;

- canned assistant answers;

- one excise jurisdiction with placeholder rates.

Out of scope: real connectors, authentication beyond a role switcher, real ML training, billing, mobile apps, APS planning, taste forecast (shown as a “coming later” card only).

Fixed rules:

- Read only. No screen offers to change setpoints or control equipment.

- Labels. Automatic events carry a “detected” label; user entries show the user name.

- Compliance wording. Quality documents read “Prepared by DataVest Brew · Released by [signer]”.

- Placeholders. All limits and rates are illustrative and marked as such.

## Demo tenant

| Item | Synthetic setup |
| --- | --- |
| Brewery | “Alder & Pike Brewing”, one site, mid-sized |
| Vessels | 12 fermenters (FV-01 to FV-12), 4 bright tanks (BBT-01 to BBT-04), brewhouse, packaging line |
| QWX43 | On 8 fermenters, via simulated Netilion |
| PLC feeds | OPC UA on all fermenters (temperature, setpoint, pressure, valve state); Modbus TCP on bright tanks; Profinet on the brewhouse |
| E+H sensors | pH on the brewhouse; dissolved oxygen and CO₂ on the packaging line |
| Recipes and tech cards | 6: Pilsner, Helles, Pale Ale, IPA, Stout, Non-alcoholic Lager |
| Yeast | 3 strains (lager, ale, NA strain) with 1–8 generations |
| History | 90 days, 95 batches (8 open), with CIP events, lab results, tasting panels and packaging releases |
| Excise | One jurisdiction, monthly declaration, rate per hectolitre and degree Plato (placeholder values) |
| Users | Head brewer, cellar operator, lab technician, quality manager, technologist, production manager, finance |

## Demo anchors

Each anchor is a seeded situation that a screen must show without setup.

| ID | Situation | Where it shows |
| --- | --- | --- |
| A1 | FV-07: fermentation running long; density nearly flat for 3 days after a slow start | Tank board, batch view, alert, assistant |
| A2 | FV-03: programmed diacetyl rest (temperature step up); not flagged | Batch view (rest shown on the plan line) |
| A3 | FV-05: CIP detected automatically from media change; detergent not yet confirmed | CIP log, task list |
| A4 | QWX43 on FV-02: accuracy check overdue; last Offset 4 months ago | Sensor accuracy, alerts |
| A5 | Lager strain: viability falling from 96% (gen 1) to 82% (gen 6) | Yeast management |
| A6 | Pale Ale batch: VDK above specification; release blocked | Packaging release |
| A7 | Profinet gateway on the brewhouse offline for 40 minutes; data buffered and backfilled | Connections, alerts |
| A8 | Non-alcoholic Lager: alcohol below limit throughout, lab result attached, quality document ready | Packaging release, reports |
| A9 | Monthly excise declaration draft for the last month | Reports and excise |
| A10 | Forecast: FV-09 ready tomorrow around noon, with range | Tank board, batch view |

## Navigation and screens

Navigation groups: Operate, Lab and quality, Production setup, Equipment, Reports. A role switcher in the header changes the default landing screen.

| ID | Screen | Purpose | Key elements | Anchors |
| --- | --- | --- | --- | --- |
| S01 | Tank board | Status of every vessel at a glance | Grid and list views; tile shows stage, extract, alcohol, temperature against plan, forecast end with range, trust indicator, alert marker | A1, A10 |
| S02 | Batch view | One batch in full | Curves (extract, alcohol, temperature, sound velocity) with plan and recipe envelope; events (fill, rests, dry hop, transfer); lab points; tech-card stage; forecast band; actions log | A1, A2, A10 |
| S03 | Alerts and tasks | What needs attention | Alerts with severity, evidence and suggested checks; production tasks with assignee and due date; acknowledge and close | A1, A3, A4, A7 |
| S04 | CIP log and schedule | Cleaning history and plan | Detected CIP events per vessel; detergent confirmation by QR or archive; schedule with overdue marks | A3 |
| S05 | Lab overview | Lab work in one place | Pending samples; latest results; out-of-spec list | A6 |
| S06 | Yeast management | Strains and generations | Strain cards; viability and cell-count chart by generation; repitch status | A5 |
| S07 | Microbiology | Plating and PCR | Test log per batch; results; links to batches | A6 |
| S08 | Tasting panel | Blind sensory evaluation | Panel setup; scores per taster and attribute; off-flavour ticks; aggregate | A6 |
| S09 | Packaging release | Decide whether a batch can be packaged | Checklist from specifications; lab, micro and sensory status; release or block with reason; quality document preview | A6, A8 |
| S10 | Tech cards and recipes | Define how a beer is made | Raw-material norms; stages with temperature profile; specifications; limit profile | A2 |
| S11 | Batch and raw-material entry | Record a batch | Form with archive picks and QR scan; error checks (empty fields, units, ranges, duplicates) | — |
| S12 | Connections | Equipment connectivity | Gateway status; protocol feeds (OPC UA, Modbus TCP, Profinet, Netilion, E+H); tag mapping wizard; buffer and backfill status | A7 |
| S13 | Sensor accuracy | Offset and accuracy checks | Per-device Offset log with reference-wort form; accuracy checks (air, distilled water) with due dates; trust score history | A4 |
| S14 | Reports and excise | Outputs | Batch and monthly quality reports; quality documents; excise declaration draft and export | A8, A9 |
| S15 | Assistant | Questions in plain language | Chat with cited records; suggested questions | A1, A5 |

## Entities for the prototype

The prototype stores data as JSON collections with these entities. Field lists are minimal; the architecture tab holds the full model.

| Entity | Key fields |
| --- | --- |
| Vessel | id, name, type, volume_hl |
| Device | id, vendor, model, serial, protocol (netilion, eh_system, opcua, modbus, profinet), vessel_id, trust |
| Tag | id, device_id, parameter, unit, address |
| Reading | ts, tag_id, value, quality, batch_id, stage |
| TechCard | id, recipe, version, raw_material_norms[], stages[{name, temp_c, duration_h, tolerance_c}], specs[], limit_profile |
| Batch | id, number, tech_card_id, vessel_id, status, opened_at (detected), closed_at |
| RawMaterialLot | id, material, supplier, supplier_lot, quantity, unit, batch_id |
| CipRecord | id, vessel_id, start, end, media_detected[], detergent, operator, status |
| OffsetRecord | id, device_id, date, wort_composition, lab_values, correction |
| AccuracyCheck | id, device_id, medium (air, water), result, tolerance, due_next |
| YeastGeneration | id, strain, generation, cell_count, viability_pct, source_batch_id |
| LabResult | id, batch_id, parameter, value, unit, method, spec_min, spec_max |
| MicroTest | id, batch_id, type (plating, pcr), organism, result |
| TastingPanel | id, batch_id, scores[{taster, attribute, score}], off_flavours[] |
| ProductionTask | id, type, target, assignee, due, status |
| PackagingRelease | id, batch_id, checks[], decision, signer, ts |
| ExciseDeclaration | id, period, lines[{batch_id, volume_hl, plato, rate, amount}], status |
| Alert | id, rule, severity, target, evidence[], state |
| Forecast | batch_id, end_ts, band_low, band_high, model |

## Rules and formulas

All thresholds are placeholders for the demo.

| Rule | Logic |
| --- | --- |
| Degree of fermentation | (original extract − residual extract) / original extract |
| Alcohol | Device value from the QWX43 feed; no recalculation in the prototype |
| End of main fermentation | Density change below 0.1 °P per 24 h for at least 24 h |
| Slow fermentation (A1) | Extract decline below 40% of the recipe envelope for 24 h → warning; 48 h → critical |
| Temperature rest versus fault (A2) | Compare temperature with the tech-card stage profile ± tolerance; inside the plan → no alert |
| Media change (A3) | Sound-velocity jump above a site threshold within 10 minutes → classify medium by configured bands → open CIP record or batch |
| Accuracy check due (A4) | Air and water check every 30 days, Offset review every 90 days (recommended defaults, configurable) |
| Yeast viability (A5) | Warning below 90%; critical below 85%; maximum generation per strain from the tech card |
| Release block (A6) | Any lab value outside specification or a positive micro test disables release and names the reason |
| Gateway offline (A7) | No data from a feed for 5 minutes → alert; on reconnect, buffered data is backfilled and the alert resolves |
| Limit profile (A8) | Non-alcoholic profile: alcohol ≤ 0.5% (placeholder); lab result required before release |
| Excise (A9) | amount = volume_hl × original extract (°P) × rate per hl·°P (placeholder rate) |
| Forecast (A10) | Logistic fit to extract over time; band widens before 50 h from pitch |

## Simulated integration

- Gateway: one simulated edge gateway with status (online, degraded, offline), buffer fill and last sync time.

- Feeds: OPC UA (fermenters), Modbus TCP (bright tanks), Profinet (brewhouse), Netilion (QWX43), E+H system (pH, oxygen, CO₂). Each shows connected tags and last value.

- Tag mapping wizard: pick a feed, browse tags, map to vessel and parameter, confirm units. In the prototype the browse list is synthetic.

## Assistant: canned answers

Every answer cites records by ID.

- “Why is FV-07 slow?” — slow start, low lager-strain viability (gen 6), similar past batches; suggests checking original gravity and pitch rate.

- “When will FV-09 be ready?” — forecast with range.

- “Which sensors need an accuracy check?” — lists FV-02 and due dates.

- “How is the lager strain doing?” — viability trend by generation.

- “Why is the Pale Ale batch blocked?” — VDK above specification, with the lab record.

- “What happened to the brewhouse data yesterday?” — Profinet gateway outage and backfill.

- “Show last month’s excise.” — the declaration draft total.

- “Which tanks were cleaned today?” — CIP records, including the unconfirmed one.

## Synthetic data generator

The existing generator is extended to produce:

- QWX43 series (density, viscosity, sound velocity, temperature and derived values) with realistic fermentation curves, temperature rests and media changes;

- PLC tag series per protocol, with one Profinet outage and backfill;

- E+H pH, oxygen and CO₂ series on the brewhouse and packaging line;

- CIP records aligned with detected media changes;

- Offset records and accuracy checks, one overdue;

- yeast generations with declining viability;

- lab results, micro tests and tasting panels, including one out-of-spec batch;

- packaging releases, quality documents and one excise declaration draft.

Output: JSON collections for the front end, plus CSV time series and a loader for PostgreSQL with a time-series extension.

## Demo script (15 minutes)

- Connections: show the gateway and the five feeds; map a tag in the wizard.

- Tank board: point to FV-07 (slow) and FV-09 (forecast).

- Batch view FV-07: curves, plan line, alert evidence; ask the assistant why.

- Batch view FV-03: the diacetyl rest that is not flagged.

- CIP log: the detected clean on FV-05; confirm the detergent by QR.

- Sensor accuracy: overdue check on FV-02; Offset log.

- Yeast management: viability falling over generations.

- Packaging release: the blocked Pale Ale and the released non-alcoholic Lager with its quality document.

- Reports: last month’s excise draft.

- Close on the “coming later” cards: APS planning and taste forecast.

## Visual direction

- Typography: Space Grotesk for numbers and headings, IBM Plex Sans for UI text.

- Colour: neutral surfaces with one accent colour, to be taken from the DataVest logo.

- Status families: device health (NE 107), alert severity and release state use separate colours and are never mixed.

- Vessel tile: fill level shows fermentation progress; one priority marker per tile.

- Forecasts: always shown with a range.